# Interoperable IoT Architectures – Web of Things

The purpose of the WoT Working Group is to counter the fragmentation of the IoT through the specification of building blocks that enable easy integration of IoT device and services across IoT platforms and application domains.

Terminology W3C

● Thing Description (TD): Structured data describing a Thing,  comprises general metadata, domain-specific metadata, Interaction Affordances ,and links to related Things.

● Binding Templates: A re-usable collection of blueprints for the communication with different IoT platforms. The blueprints provide information to map Interaction Affordances to platform-specific messages through WoT Thing Description as well as implementation notes for the required protocol stacks or dedicated communication drivers.

● Consumed Thing: A software abstraction that represents a remote Thing used by the local application. The abstraction might be created by a native WoT Runtime, or instantiated as an object through the WoT Scripting API.

● Consuming a Thing: To parse and process a TD document and from it create a Consumed Thing software abstraction as interface for the application in the local runtime environment.

● Exposed Thing: A software abstraction that represents a locally hosted Thing that can be accessed over the network by remote Consumers. The abstraction might be created by a native WoT Runtime, or instantiated as an object through the WoT Scripting API.

● Thing Directory: A directory service for TDs that provides a Web interface to register TDs (similar to [CoRE-RD]) and look them up

Common Patterns

Have the role to illustrate how device/things interact with controllers, other devices, agents and servers

- Client role as the initiator of a transport protocol

- Server role as passive

- A device can be client and server simultaneously

Pattern  Device controllers

Controlled by user-operated remote controller

- In this pattern there is an electronic appliance that has server role that can accept request from the other devices and responds to them.

- The other device is the remove controller that has client role (browser or native app), can send request like to read sensor value or turn on a device.

Pattern  Thing to Thing

A sensor detect a change on the room condition and notify to turn on the electronic appliance

- The sensor can notify a change of state to other devices

- In this case two devices have the server role so at least one device must also have a client role.

Pattern  Remote Access

The remote controller can switch between different network and protocols.

- When the controller is in the local network it is a trusted device and no additional  networking security or access control is required

- Outside instead is not a trusted network

- In this case the remote controller and the electronic appliance have the a client and a server role.

Pattern  Smart Home Gateway

The gateway is placed between a local network and the internet.

Can Receive commands

It is also a virtual representation of a device.

Typically offers proxy  and firewall functionality.

The gateway cover both client and server role.

The gateway act as a server for the appliance and as a client for the remove controller.

Pattern  Edge Devices

While the gateway work as bridge, the edge device has local compute capabilities and typically bridges between different protocols

Edge device can provide preprocessing, filtering  and aggregation of data provided by connected devices and sensors

Pattern  Digital Twins

Is a virtual representation, model of a device or a group of devices that resides on a cloud server or edge device

- Can be used to represent real-world devices which may not be always online.

Cloud Ready devices

In this patterns  appliance are directly connected to the cloud.

The cloud mirrors the appliances and, acting as a digital twin.

Authorized controllers can be located anywhere.

Legacy Devices

In this case cannot directly connect to the cloud.

The gateway is needed to relay the connection and works as:

Integrator of protocols

- Firewall

- Privacy

- Local agent (in case of network connection disruption)

- Emergency Service (Running locally)

The cloud mirrors the gateway with all connected appliance and act as a digital twin and also receive command from remote controllers

Pattern  Multi Cloud (More than one cloud)

Pattern  Cross-domain Collaboration

In this pattern each system involves and interact with other systems in other domain. This type of system is called Symbiotic ecosystem

- Direct Collaboration: model, systems exchange info in a peer-to-peer manner

- Indirect Collaboration: systems exchange info via some collaboration platform.

W3C Wot Architectural Common Principles

Flexibility, Compatibility, Scalability and Interoperability

W3C Wot _Things Functionalities

Reading, updating, subscribing, invoking functions, subscribing to event notification

WoT Support also a mechanism which enables describing things and their functions:

- Should not be only human-readable, but also machine-readable.

- Should allow semantic annotation

- Should be able to be exchanged using multiple formats

- Should support also internalization

Attributes described such as name, explanation, version, format and description, link to other related things.

WoT  Devices

Access to devices is made using a description (TD) of their functions and interfaces

The Thing Description includes a general metadata about the deice, information models, representing functions, transport protocol description, and security info.

General metadata contains device identifiers (URI), device info such as serial number, production date, location and other human readable info.

Info models defines device attributes, and represent device’s internal settings, control functionality and notification functionality.

Security info includes description about authentication, authorization and secure communications.

TD must be accessible to other components.

WoT  Applications

Need to be able to generate and use network and program interfaces based on metadata and descriptions.

Must be able to obtain these descriptions, therefore need t e able to conduct search operations.

WoT  Digital Twins

Need to generate program interfaces internally based on metadata.

A Twin has to produce a description for the virtual device.

Id of vt devices must be newly assigned, therefore, are different from the original devices.

Transport and security mechanism and settings can be different from the original devices if necessary

WoT  Discovery

Directory can serve the requirement of sharing TDs so devices and twins can automatically or allow the users to register the TD.

Descriptions of the devices need to be searchable.

WoT  Architecture - Affordances

A central aspect is to provide machine-understandable metadata

A key to this self-descriptiveness lies in the concept of affordances

Affordance refers to the perceived and actual properties of the thing, primarily those fundamental properties that determine just how the thing could possibly used.

- Descriptions: In the TD the affordances must be described.

- Interoperability

- Communication & Interaction: allow WoT devices to communicate and interact with each other

- Security & Authorization

Behavior aspect of a Thing includes both the autonomous behavior and the handlers for the Interaction Affordance.

The Interaction affordances provide a model of how consumers can interact with the thing.

Data schemas describes the info model and the related payload structure and the items passed between the Things and Consumers during interactions.

The Protocol Binding add the additional detail needed to map any interaction with a message of a determinate protocol

The security configuration aspect of a thing are the mechanism used to access the Interaction affordance.

Protocol binding:

Thing Description is based on JSON

Example of a TD:

Things Linking

Linking do not apply hierarchical Things but relations between things and other resources.

System components and their Interconnectivity

Direct communication applies when both Servients use the same network protocol(s) and are accessible to each other

In a Thing, a Servient software stack contains a representation of a Thing called Exposed Thing

It makes its WoT interface available t Consumers of the Thing

Intermediary is yes another WoT architecture implemented by a servient like the consumer

Is located between a Thing and its consumers.

Consumer and Intermediary can communicate in a different protocol than Intermediary and Thing.

The purpose can be relaying, optionally caching or transforming communication.

Take a look to discovery approach and use case od WoT last 20 pages.